Harmonized calculation method 
for Africa 
(HCM4A) 
USER GUIDE for the gui program 
for MOBILE service

Contents

1.	GENERAL	3

2.	CalsMS4A OVERVIEW	3

3.	INTERFERENCE CALCULATIONS	4

3.1 Point to point calculations	5

3.2 Line calculations	7

3.3 CBR calculations	9

3.4 Results combining (preparing response)	10

4.	OPENING AND EDITING OF FILES (LISTS)	11

5.	CALCMS4A OPTIONS	13

6.	UPDATE OF THE GUI PROGRAM AND DLL	14

7.	INSTALLER	14

ANNEX. Error codes.	15

# GENERAL

This harmonized calculation method for the mobile service is part of the HCM4A Agreement.  The GUI program, empowered by the calculation library (DLL) provides means to assess interference between mobile stations in point-to-point manner (relevant mostly to PMR/PAMR spectrum use), calculate interference into neighboring countries at cross-border range locations and also along x-km lines equidistant from the border of a country, based on the stations descriptions in the file format following Annex 2 of the Agreement, including text based, Excel and JSON versions of the format.

HCM4A software for mobile service consists of two parts – the calculation library (hcm4ams.dll) and the program (CalcMS4A). The implementation of both reuses many elements from the similar software developed for the HCM Agreement in Europe, but implemented independently for the HCM4A Agreement. However, the outdated elements of Fortran90 code from the HCM software had been remade into more readable and controllable С++ functions. All calculations are being made in the calculation library. CalcMS4A does not make calculations by itself; instead, it uses the library for calculations. The GUI program just performs sequential call of the library main subroutines to get results for multiple stations, but it provides some automation in switching between interference criteria in accordance with the HCM4A Agreement.

This document serves as a user’s guide to the GUI program. Although the HCM4A software is developed for Microsoft Windows operating system, it may also be compiled for Linux or MacOS. In case a calculation result for Microsoft Windows differs from the version for other OS, version of the HCM4A for mobile service for Windows OS will be the reference.

General note:

In this description (unless stated differently):

all angles are in degrees,

all heights are in meters,

all distances are in kilometers.

# CalsMS4A OVERVIEW

CalsMS4A is created to help Administrations perform interference calculations according to HCM4A Agreement. In addition to that main purpose, it allows creating, editing and checking HCM4A formatted files. CalsMS4A graphical user interface is built around one main window with multiple tabs used for calculation.

The program includes following tabs:

- Test List, used to upload new stations such as a file of stations from a country requesting coordination;

- Reference List, used to upload existing (already coordinated and protected) stations of a country responding to the coordination request (this tab is only used for point to point calculations);

- Maps, used to visualize positions of stations and some calculation results on the geographical map;

- Calculations Point to Point, used to control and initiate calculations in the point to pint mode;

- Calculations Line, used to control and initiate calculations in the point to line mode;

- Calculations CBR, used to control and initiate calculations in the cross border range mode;

- Combine results, used to prepare a file to respond the coordination request based on calculation results.

The process of coordination calculation consists of uploading Test List stations and in some cases reference stations, proceeding with specific calculation and finally preparing an output file to be used in the response. Specifics of interference calculation modes are described in section 3. Uploading and editing of files are reviewed is section 4. Sections 5 to 7 describe installing and configuring of the software.

The program is cross-platform. Although the main platform for compilation is Windows, it may also be available for Linux and MacOS. The program is created using C++ and U++ framework.

# INTERFERENCE CALCULATIONS

CalcMS4A performs three types of interference calculations:

- Point to point calculations. In this case user has to provide CalcMS4A with two files. One has to uploaded as a test list of stations, another – has to be uploaded as the list of reference stations. This mode calculates interference from Tx or TRx in the test list into receivers in the reference list (Rx or TRx). It could be used also for single case calculations (one Tx in the test list and one Rx in the reference list).

- Cross-border range calculations. In this case user has to provide CalcMS4A with a file as a test list of stations (Tx or TRx more specifically, Rx are ignored). The calculation also requires the user to specify the country to which interference will be calculated (the originating country is assumed in Tx station records) and to specify cross-border range distance (depth of interference into another country along radius from Tx station).

- X-line calculations. In this case user has to provide CalcMS4A with a file as a test list of stations (Tx or TRx more specifically, Rx are ignored). The calculation also requires the user to specify the country to which interference will be calculated (the originating country is assumed in Tx station records) and to specify the x-km equidistant line from the border inside another country (including 0-km line, e.g. border itself).

Interference calculation are based on the library, which could return error codes in many cases of inappropriate input data. These codes are listed in the Annex and shown in results to guide the user in adjusting input data to avoid incorrect values of parameters.

## 3.1 Point to point calculations

Point to point calculations perform analysis of one file entries against another. Before performing calculations, you need to set appropriate paths required by CalcFS4A calculation library (hcm4ams.dll). See section on setting up path further in the document. Each file is uploaded in the relevant tab.

The results of the upload of relevant stations could be observed on the map tab, which also automatically upload existing borders on the map (files with “.ALL” extensions from BORDER folder, see details below). The map also shows radius of service area for mobile station or azimuth, if station is fixed. Keys combinations for distance measurement, terrain and morpho layers visualization are listed in the left bottom corner. This functionality is not used in calculations, but may be helpful in investigating specific interference cases.

After the upload of stations is finished and their positions are checked with the map, the calculation is done from the Calculations Point to Point tab. This tab has control elements to initiate calculation for point-to-point calculation type and form to check calculation results.

For point-to-point calculations, one should select one of four modes of calculations:

0: P2P (normal HCM Agreement) with t%=1 or 10, depending on Tx channel occupancy;

+10: P2P with t% = 10

+11: P2P with t% = 50

+12: P2P with t% = 1

Besides mode of calculation, it is necessary to specify base permissible field strength (based on Annex 1 of the Agreement or chosen any other way) and depolarization loss. Taking into account the depolarization loss is optional and should be agreed for individual coordination requests on a bilateral basis.

If the existing field “depolarization loss” is set to -9.9 the HCM program for P-P calculation considers one station as vertical polarized and the other as horizontal and calculates according the following principles:

If both transmitter and receiver are base stations, and if the antenna of the one station is vertically polarized, while that of the other station is horizontal polarized, and  if the gain losses of both antennas each are less than 10 dB, a correction for the depolarization decoupling is to be considered according to the following equation:

aDepol = 25dB - 0.5 * (F0-F), in the area of 0 to 50dB for (F0-F) and

aDepol = 0 for (F0-F) > 50dB

With:

F0 	= free space field strength at the receiving site in dBµV/m

F 	= predicted field strength at the receiving site in dBµV/m

aDepol 	= depolarization decoupling loss in dB

The calculation results are then presented in the same tab. Note that permissible FS is adjusted in point-to-point calculations taking into account spectrum masks and antenna pattern of Rx for each particular pair.

The result of calculations could be saved as an Excel file, but its main purpose is to be used for preparing a response in the combine results tab. The result is automatically transferred to the combine results tab. There is also a button to clear calculations results in case some modifications are introduced in the input data (not required in usual assessment of coordination requests).

## 3.2 Line calculations

Line calculation perform analysis of Tx entries in the test list only, ignoring Rx in this list or any records uploaded in the reference list tab.  For line calculations one should provide the country to which interference is directed and assessed as well as the x-km distance to select a line from existing preset (6, 9, 15 km and etc.) together with the permissible FS (for example from ECC recommendations or bilateral/multilateral agreements). It should be noted that no corrections are automatically provided to the permissible FS for line calculations, except optional correction in relation to 5 MHz bandwidth (activated by a separate checkbox). Finally, one should select an appropriate mode of calculation, e.g. for ECC Recommendations it would be mode -9 with 3 meters antenna height of the receiver and t% = 10.

After pressing the calculation button for the line type calculations, the result will be provided in the table below. Each line of results could be also used to show more detailed calculation log for each of transmitters. By double clicking on a line of result a text file will appear with intermediate calculation results and all input parameters used in the calculation process.

It should be noted that HCM4A subroutines (similar to HCM counterpart) provides results only for worst case point on the x-km line and do not provide full details how this point has been derived (from iterative search inside the library). The final points from the search algorithm could be observed on the map tab together with x-line visualization.

## 3.3 CBR calculations

Cross-border range calculations are implemented very similarly to Line calculations. For CBR calculations one should provide the country to which interference is directed and assessed to select proper .CBR file (the segment of the border of the country sending coordination request) together with the permissible FS. It should be noted that no corrections are automatically provided to the permissible FS for CBR calculations. Finally, one should select an appropriate mode of calculation, usually -1 for normal HCM4A Agreement calculation.

By default, only Tx records are considered within the test list. To apply an automatic conversion of receivers (and transceivers) into equivalent(additional) transmitters a separate checkbox should be activated. The test list control elements are shown below.

After pressing the calculation button for the CBR type calculations, the result will be provided in the table below.

Each line of results could be also used to show more detailed calculation log for each of transmitters (see example in the previous section).

It should be noted that HCM4A subroutines (similar to HCM counterpart) provides results only for worst case point among CBR generated locations and do not provide full details how this point has been derived (from iterative search inside the library). The final points from the search algorithm could be observed on the mapы tab.

## 3.4 Results combining (preparing response)

Multiple calculation modes create some complexity on the way how to represent results for the response in the cross-border negotiations. To resolve this complexity a separate tab for results combining had been proposed. This tab, shown below, provides way to combine result from the methods of calculations.  However, it should be noted that usual HCM4A practice will include either combining P2P and CBR modes, or using Line calculations as a standalone method. The example below is based on the combination of P2P and CBR results, which had been calculated using previously described tabs. By pushing on the update station file button, the new temporary file is created with field 13Y and 13Z modified according to the rule shown in the tab.

In addition to just using permissible field strength from the calculation the tab also provides means to offset interference, for example ignore interference exceedance of 1-2 dB. As P2P interference is exceeded, 13Y field is written with “Z” status. The status for positive or negative result is controlled by the tab. 13Z field is overwritten with the short summary of interference across all modes (only P2P exceedance in the example.)

The interface also provides fields to properly fill in the header of the file as the response is now prepared by a new person in another country and the destination country and the date are updated as well. See details in Annex 2A of the HCM4A Agreement.

After all header fields are in order the output file could be saved by pressing the save to file button. It could be saved in a text, Excel or JSON formats. If further adjustments in response are required, they should be done manually by opening such file in the Test List or the Reference List tabs.

# OPENING AND EDITING OF FILES (LISTS)

To upload station records to the test list or to the reference list the button “Open” should be used, which will open up a dialog box to find proper txt file. The interface is similar for the Tesl List tab and for the Referene List tab.

Text files should be properly formatted in accordance with Annex 2A of the HCM4A Agreement. Also, equivalent Excel and Json formats are supported.  The file should also include header. An example of the same file in text and Excel format is shown below.

Opening a new file will automatically clear any existing calculations results to avoid mismatch between input data and output results.

To be able to create the test list or reference list from multiple files, the “Add stations from file” could be used. The difference of this button compared to the “open” button is that it doesn’t delete existing stations and it skips the header of the file (the header of the initial file is retained). The “Save” button provides functionality to save compiled list into a new file (operates as “Save as”). Buttons “Delete One” and “Delete All” operates as the name suggest.

For detailed editing of separate records, one should double click the entry of specific record in the test list or in the reference list, which will open an editor for specific records either in the test list or in the reference list.

The editor provides direct access to all fields in Annex 2A format. The upper part shows the header similar to all stations in the list (but different for the test list and the reference list accordingly). The editor has two buttons of its own. The delete button operates the same way as the “Delete” button from the list tab. The “Add or copy” button copies the current record or creates a new one if the list is empty (usually it much easier to modify existing one rather that to create one from scratch).

It is important to note that after modifying any parameter, these modifications need to be saved by the save station button in the lower part of the editor. Otherwise, these changes will be lost.

# CALCMS4A OPTIONS

In order to work properly CalcMS4A requires several paths set correctly. The menu to set path includes three entries. All three folders are mandatory for CalcMS4A. This form also contain special global parameter to limit (skip) calculations with distance beyond the limit. African continent has a number of countries with very long mutual borders, which could lead to extensive calculations in case of multiple stations. Therefore, this limit forces the software to skip such points and focus only on shorter interference paths (but not shorter than 200 km as double of CBR maximum value for lower frequencies)

# UPDATE OF THE GUI PROGRAM AND DLL

For simplicity of deployment the update is done by downloading updated version of the GUI program, overwriting existing files. The version of the program is reflected in the name of the main window. The version of the calculation library is shown in the detailed log of calculations.

# INSTALLER

For simplicity of deployment the GUI program is distributed as a portable software, meaning it is enough to copy a folder with the software (and to properly set paths to topographical and morphological data) to the computer to start using it without any need for traditional installation. If required the installation setup could be created, but the software doesn’t require any specific permissions or register record to operate properly anyway.

# ANNEX. Error codes.

| 0 | No error |
| --- | --- |
| 36 | Error opening terrain- or morphological data file (data not available) |
| 200 | Error in longitude (in 'Point_height' or 'Point_type' subroutine) |
| 210 | Error in latitude (in 'Point_height' or 'Point_type' subroutine) |
| 220 | Error reading record (in 'Point_height' or 'Point_type' subroutine) |
| 300 | Latitude is not in range of 0.0 - 90.0 (in 'Point_height' or 'Point_type' subroutine) |
| 400 | Height is missing (-9999) (in 'Point_height' subroutine) |
| 1000 | Distance between Tx and Rx = 0. Calculations not possible |
| 1001 | Error in geographical coordinates (Tx longitude, degrees) |
| 1002 | Error in geographical coordinates (Tx longitude, minutes) |
| 1003 | Error in geographical coordinates (Tx longitude, seconds) |
| 1004 | Error in geographical coordinates (Tx longitude, E/W) |
| 1005 | Error in geographical coordinates (Tx latitude, degrees) |
| 1006 | Error in geographical coordinates (Tx latitude, minutes) |
| 1007 | Error in geographical coordinates (Tx latitude, seconds) |
| 1008 | Error in geographical coordinates (Tx latitude, N/S) |
| 1009 | Error in Tx antenna height |
| 1010 | Error in transmitting frequency value |
| 1011 | Error in transmitting frequency unit |
| 1012 | Error in radius of service area of Tx |
| 1013 | Error in input value height of Tx site above sea level |
| 1014 | Error in geographical coordinates (Rx longitude, degrees) |
| 1015 | Error in geographical coordinates (Rx longitude, minutes) |
| 1016 | Error in geographical coordinates (Rx longitude, seconds) |
| 1017 | Error in geographical coordinates (Rx longitude, E/W) |
| 1018 | Error in geographical coordinates (Rx latitude, degrees) |
| 1019 | Error in geographical coordinates (Rx latitude, minutes) |
| 1020 | Error in geographical coordinates (Rx latitude, seconds) |
| 1021 | Error in geographical coordinates (Rx latitude, N/S) |
| 1022 | Error in Rx antenna height |
| 1023 | Error in reception frequency value |
| 1024 | Error in reception frequency unit |
| 1025 | C_mode is out of range |
| 1026 | Error in input value of permissible field strength |
| 1027 | Error in input value of maximum cross border range |
| 1028 | The distance is greater than permitted*. Calculations not possible |
| 1029 | Error in radius of Rx service area |
| 1030 | Error in input value Rx site height above sea level |
| 1031 | Error in Tx elevation |
| 1032 | Error in Tx azimuth |
| 1033 | Error in type of Tx antenna (E/I) |
| 1034 | Error in power |
| 1035 | Error in input value of distance over sea |
| 1036 | The 'xxx.ALL' borderline file for Tx is missing |
| 1037 | The 'xxx.ALL' borderline file for Rx is missing |
| 1038 | Error in type of antenna |
| 1039 | Error in input data of correction factor according frequency difference |
| 1040 | Channel spacing outside definition range (Rx) |
| 1041 | Channel spacing outside definition range (Tx) |
| 1042 | Error in Rx elevation |
| 1043 | Error in Rx azimuth |
| 1044 | Error in Rx type of antenna ("E" or "I") |
| 1045 | Error in gain of Rx antenna |
| 1046 | Error in input data of depolarization loss |
| 1047 | Distance to borderline is too long |
| 1048 | Selected line data not available |
| 1049 | Error in line data |
| 1050 | No HCM Agreement frequency and important technical data missing e.g. CBR, max. perm. FS |
| 2000 | wrong Figure_frequency (from Get_figure_FS_value) |
| 2001 | wrong Time_percentage (from Get_figure_FS_value) |
| 2002 | wrong Sea_temperature (from Get_figure_FS_value) |
| 2003 | wrong Figure_Heff (from Get_figure_FS_value) |
| 2004 | wrong Figure_distance (from Get_figure_FS_value) |
| 3000 | DLL input string to short |

* - the distance is a parameter established in the GUI and transferred to the library