Harmonized calculation method 
for Africa 
(HCM4A) 
USER GUIDE for the gui program 
for FIXED service

Contents

1.	GENERAL	3

2.	MAIN HCM4A CALCULATION PROGRAM FOR FIXED SERVICE (CalsFS4A)	3

3.	INTERFERENCE CALCULATIONS	4

3.1	List to list calculations	5

3.2	Single entry calculations	10

4.	TRIGGER FOR CO-ORDINATION	12

5.	CALCFS4A OPTIONS	13

5.1	Paths	13

5.2	Calculation	13

5.3	Control of input parameters	14

6.	FILE EDITING	14

7.	UPDATE OF THE GUI PROGRAM AND DLL	18

8.	INSTALLER	18

# GENERAL

This harmonized calculation method for the fixed service is part of the HCM4A Agreement.  The GUI program, empowered by the calculation library (DLL), provides means to assess interference between fixed point-to-point links, which may also include one passive back-to-back antenna repeater, described in the file format following Annex 2B of the Agreement.

HCM4A software for fixed service consists of two parts – the calculation library (hcm4afs.dll) and the program (CalcFS4A). The implementation of both reuses many elements from the similar software developed for the HCM Agreement in Europe, but implemented independently for the HCM4A Agreement. All calculations are being made in the calculation library. CalcFS4A does not make calculations by itself; instead, it uses the library functions calls for calculations. However, the detailed procedure how to account all potential combination of interference paths through passive repeaters are described within the GUI program. This document serves as a user’s guide to the GUI program. Although the HCM4A software is developed for Microsoft Windows operating system, it may also be compiled for Linux or MacOS. In case calculation result for Microsoft Windows differs from the version for other OS, version of the HCM4A for fixed service for Windows OS will be the reference.

General note:

In this description (unless stated differently):

all angles are in degrees,

all heights are in meters,

all distances are in kilometers.

# MAIN HCM4A CALCULATION PROGRAM FOR FIXED SERVICE (CalsFS4A)

CalsFS4A is created to help Administrations perform interference calculations according to HCM4A Agreement. In addition to that main purpose, it also allows creating, editing and checking HCM4A formatted files.

Calculation library performs two types of calculations:

- Interference calculations. The main output parameter is Threshold degradation (TD).

- Calculations of co-ordination trigger, performed according to Annex 11 to the HCM-Agreement. This may help the administrations to decide if stations require co-ordination. The main output parameter is name of countries to coordinate with.

The GUI program code is cross-platform. Although the main platform for compilation is Windows, it may also be compiled for Linux and MacOS. The program is created using C++ and U++ framework.

The underlying library and the program attempt to perform calculations and return error code for those cases, it can’t process properly. These codes are listed in Annex 1.

The main program window, called by CalsFS4A.exe, is used to navigate between major more specialized windows and forms. The main window is shown below in include number of buttons and menus to initialize different parts of the software.

The first button from left to right invokes an editor window, which is used to prepare or manipulate single file, see section 6 for details. The second button opens up the interference calculation window, explained in section 3. Trigger for co-ordination is called by the third button to check distances to other countries borders (see section 4). The fourth button is used to open up the configuration menu, explained in section 5. The fifth button shows the information about the software. The text menus are duplicating the same functionality with only difference that the calculation menu includes two sub-options for interference calculations and for the coordination trigger check.

# INTERFERENCE CALCULATIONS

Called by  button from the main window, CalcFS4A shows the interference calculation window to perform one of two types of interference calculations:

- List to list calculations(major type of calculations). In this case user has to provide CalcFS4A with two files. One has to contain reference list of stations, another – the list of stations being tested. This is the main mode of the software to be used for cross-border coordination interference analysis.

- Single entry calculations(used for testing purposes). Program calculates interference just for a single case. User has to provide the program with the file containing either 2 or 4 records depending on whether passive repeater is present. This mode is mainly used for testing.

The calculation window shown below includes following buttons for:  Single entry calculations,  List to list calculations, File validation, Map visualization, Configuration and Information. Text menus provide alternative to invoke these functions. The window also includes to forms for tables representing the test list of stations and the reference list of stations, explained in the following subsections.

## 3.1 List to list calculations

List to list calculations perform analysis of one file of stations entries against another. Before performing calculations, you need to set appropriate paths required by CalcFS4A calculation library (hcm4afs.dll). See section on setting up path further in the document.

To perform the calculations, click the menu entry Calculations/List – to List Calculations or the button

. Choose HCM4A file dialog box will appear, if files haven’t been uploaded yet.

Under HCM4A testing file a file received from a neighboring country or being sent to the neighboring country should be chosen. HCM4A reference data file represents already coordinated stations, which should be protected and/or could interfere with new stations.

After clicking the button Choose HCM4A files dialog box pops up. In this case 2 file can be submitted, for example files, KEN 5-6 and ETH 7-8. These files have header fields, but header could be omitted as well.

KEN 5-6

ETH 7-8

Pressing  opens up dialog window to change some calculation parameters, simultaneously with opening and showing HCM data files as shown below. This dialog window could also star calculations or return back to stations forms by pressing cancel button.

It should be noted that TD parameter is also controlled by global settings (see section 5.2 below), but this menu gives opportunity to change it for a specific calculation without altering global settings.

After pressing the cancel button these files will be viewed in the interference calculation window. You will be able to view the data on the parameters Tx and Rx HCM data file and HCM reference data file.

In the case of passive repeater in any list a passive receiver PRX should be always followed by a passive transmitter PTX. It is recommended to place passive repeater entries between associated TX and RX, but only adjacency of PRX and PTX is mandatory.

The program allows you to represent the considered lines on the map by clicking on the button . This will open up a separate map window. Different background layers could be used in the map window by toggling different options in the upper right corner. The station tree on the left side of the map window could be used to navigate between stations.

To proceed with the calculation, you should repeat aforementioned steps, but press ok button in the calculation setting menu instead of the cancel button:

Value of Show station if TD > informs the program that all file entries which produced TD > than the value set have to be written to the report file. When set to 0 it informs program that all calculation cases have to be written to the report file. Surface temperature and pressure are taken as global average from Annex 1 of ITU-R recommendation P.835-7. Pressing ok will trigger calculation process and opening of a separate window for calculation results. Program also writes to the report cases when error occurs. The results window has two tabs: Calculation report and Results Coordination.

The calculation report tab shows details of interference calculation for each pair of transmitter and receiver under consideration with TD above the specified threshold.  By double-clicking a report entry Detailed Calculation Report appears for particular interference path.  Report for each link could be saved and opened later.

The result of coordination tab serves the purpose to prepare the response on the coordination request in a semi-automated way. It shows the proposed modified file derived from the test list file and interference calculation results. The upper part of the tab is used to enter contact details of the responding administration and the writing date. The lower part shows the contents of the test list file with the only difference in fields 13Z and 13Y. In case of interference from the Tx in the test list, 13Z will contain a short description of the situation and TD value and 13Y will have the status Z (rejecting the coordination request for this specific station). In case of interference from the Tx in the reference  list (towards new Rx), 13Z will contain a short description of the situation and TD value and 13Y will have the status G (new Rx not protected from interference).  In case of no interference, 13Z will remain empty (or with original text) and 13Y will contain status C (agreeing the coordination request.

The save button at the top of the window will open save menu to record a new file to be used for answering the coordination request. If further modifications are necessary to augment automatically generated result, the editing window should be used with this new file (see section 6).

## 3.2 Single entry calculations

The calculations are performed by clicking the menu entry Calculations/Single Entry Calculations or the button  . The program behaves very similar to the List to list calculation case. After clicking the button Choose HCM4A file dialog box pops up. In this case only one file need to be submitted, for example file, ETH 2-KEN 4.

ETH 2-KEN 4

By uploading the file, the calculation parameters will be invoked as well. By pressing the ok button the calculation process will be started, by pressing the cancel button the form with the uploaded file will remain as a current window.

It should be noted that TD parameter is also controlled by global settings (see section 5.2 below), but this menu gives opportunity to change it for a specific calculation without altering global settings.

Therefore, after pressing the cancel button, you will be able to view the data on the parameters Tx and Rx HCM4A data file. The first record in this file always must be TX, the second – RX or PRX, the third – PTX (in case passive repeater is available), the forth – RX (in case passive repeater is available).

By repeating the aforementioned steps, but pressing the ok button in setting the calculation parameters, the calculation is invoked with the calculation results being opened automatically.  In case of many stations, after calculations are started the report window will pop up showing calculation progress. Calculations produce similar report as in the List to list calculation case. Program also writes to the report cases when error occurs.

By double-clicking a report entry Detailed Calculation Report appears.

# TRIGGER FOR CO-ORDINATION

The purpose of trigger for co-ordination is to provide user with the information whether stations require co-ordination. The evaluation is performed according to Annex 11 to the HCM4A Agreement.

In order to perform such evaluation border data must be present on the computer disk and path to the border data has to be set (see the section on path further down the document). To start calculations, activate menu entry Calculations\Trigger for Co-ordination or click .

You will be asked to provide file with the stations formatted according to Annex 2B to the HCM-Agreement. For example, we will use the file KEN 14-23 G Annex11 (header is omitted). It should be noted that this button doesn’t apply the evaluation to stations in the test tab. It performs the evaluation only for stations in the file provided in a dialog box initiated by this button.

After completing the calculations using the file KEN 14-23 G Annex11, are complete the report will be shown.

The report indicates the name of the Tx and/or Rx line, the link, the error code, and the countries with which the Tx and/or Rx line needs to be coordinated. If the "Countries" field is empty, it means that the station does not need approval (it is located too far from the border of any country). The score also shows the border of his own country, which could be ignored. The report could be save to xls-file, also stations could shown on the map by pressing a dedicated button. The map window is the same as the one used in interference calculation. Pressing “Shift” button and clicking on the map it is possible to manually check distances, if required.

# CALCFS4A OPTIONS

CalcFS4A options can be accessed under main menu Tools\Options or by pressing the button . It contains all available program settings.

## 5.1 Paths

In order to work properly CalcFS4A requires several paths set correctly. The menu to set path includes three entries.

Directory of topological data is mandatory data defining path to topological data used by hcm4afs.dll. Directory of morphological data is optional and required only by those countries which are located near a sea. Directory of border data is mandatory only for evaluation of the trigger for co-ordination.

## 5.2 Calculation

There are certain triggers that prevent unnecessary calculations thus speeding up the calculation process. The form below forces the software to perform calculations, if the following conditions are met:

This menu controls only GUI filter for calculations. It should be noted that by design the calculation library may still skip calculations, specifically if frequency separation is too high and beyond spectrum masks limits.

There are two parameters which trigger calculations. It is frequency difference TX and RX, and distance between TX and RX. If both conditions are met, calculations are performed. A separate parameter to assess the need to calculate interference between a passive repeater and a potential victim receiver. The third parameter limits calculations, which include

## 5.3 Control of input parameters

Control over the values of input parameters can be enabled or disabled. This control currently provides only basic check of the type of input parameter (numerical, string, etc.).

# FILE EDITING

CalcFS4A can create a new HCM4A file for fixed service; also can edit one by changing data, adding, duplicating or deleting records.

Create a new file

To create a new file, go to File\New, or press a button.

Open an existing file

To open an existing file, go to File\Open, or press a button

Save a file

To save a file, go to File\Save, or press a button. There is a command File\Save As available, as well as a button , which always shows Save parameters dialog box. The program allows saving and making other actions on a file with an empty header. Although Save command does not show Save parameters dialog box, it also does not include HCM4A for fixed service header, if file being saved does not have one.  Buttons are also duplicated in the file menu.

By clicking on the line corresponding to a specific station a new from is opened to give access to specific field of this station as shown below.

Validating a file

The contents of uploaded file or files(List to list case) could be validated against the requirements of Annex 2B of the Agreement by pressing the button  or via the Tools menu. The result of such check is shown below.

It should be noted that not all mandatory parameters as stated in the Annex 2B of the Agreement are actually required for the calculations. The list of parameters, which are mandatory for calculations, is shown below.

| Mandatory Tx data: | Mandatory Rx data: |
| --- | --- |
| Tx Type of entry (0A) | Rx Type of entry (0A) |
| Tx frequency (1A) | Rx frequency (1A) |
| Tx frequency unit (1A1) | Rx frequency unit (1A1) |
| Tx co-ordinates (4C) | Rx co-ordinates (4C) |
| Tx height of site above sea level (4Z) | Rx height of site above sea level (4Z) |
| Tx power (8B) | Rx azimuth (9A) |
| Tx azimuth (9A) | Rx elevation (9B) |
| Tx elevation (9B) | Rx polarization (9D) |
| Tx polarization (9D) | Rx noise power level FkTB (9H) |
| Tx antenna height above ground (9Y) | Rx antenna height above ground (9Y) |
| Tx antenna gain (9X1) | Rx antenna gain (part of 9X1) |

For the following parameters default values are substituted by the program if they are not provided as input values:

Tx max. capacity of the link (7K):	X

Tx ATPC (8B3):				0.0

Tx branches and line losses (9L):		0.0

Are Tx antenna patterns present?:

If the following parameters are not provided as input value, the default values of  the HCM-Agreement are substituted for all patterns (according to antenna gain) from the file FS_DEFAULT_ANTENNA_DATA.TXT. This file has to be present in the DEFAULT_ANTENNADATA directory.

- Tx antenna copolar radiation pattern (part of 9X)

- Tx antenna crosspolar radiation pattern (part of 9X)

- Tx antenna copolar number of data (part of 9X)

- Tx antenna crosspolar number of data (part of 9X)

Is Tx spectrum mask data present?:

If no Tx spectrum mask data (7G), is present, the default data according to the HCM-Agreement is substituted.

For the following parameters default values are substituted by the program if they are not provided as input values:

Rx max. capacity of the link (7K):	X

Rx branches and line losses (9L):		0.0

Are Rx antenna patterns present?:

If the following parameters are not provided as input value, the default values of  the HCM-Agreement are substituted for all patterns (according to antenna gain) from the file FS_DEFAULT_ANTENNA_DATA.TXT. This file has to be present in the DEFAULT_ANTENNADATA directory

- Rx antenna copolar radiation pattern (part of 9X)

- Rx antenna crosspolar radiation pattern (part of 9X)

- Rx antenna copolar number of data (part of 9X)

- Rx antenna crosspolar number of data (part of 9X)

Is Rx spectrum mask data present?:

If no Rx spectrum mask data (7G), is present, the default data according to the HM-Agreement is substituted.

Add a record or delete a record

To add the record in the list of station one need to press right click on the form, listing stations records, to open a pop-up menu with such command. The same menu also has the “Delete” command to delete specific record (based on the position of the mouse cursor).

Viewing station on the map

By clicking on  button stations could be viewed on map to check consistency of coordinates provided. The first load of the map may take several minutes to upload all borders from the border folder. To reduce time delete mapping data, which is not relevant for crossborder coordination in specific country.

# UPDATE OF THE GUI PROGRAM AND DLL

For simplicity of deployment the update is done by downloading updated version of the GUI program, overwriting existing files. The version of the program is reflected in the name of the main window. The version of the calculation library is shown in the detailed log of calculations.  The contents of CalFS4A folder are shown below. Thus, updating these files with newer versions is the only action required for the update. In some cases, only executable file (.exe) or .dll may be updated, if modifications are minor.

# INSTALLER

For simplicity of deployment the GUI program is distributed as a portable software, meaning it is enough to copy a folder with the software (and to properly set paths to topographical and morphological data) to the computer to start using it without any need for traditional installation. If required the installation setup could be created, but the software doesn’t require any specific permissions or register record to operate properly anyway.

Annex. Error codes.

CalsFS4A generates the following error codes:

| 0 | No error |
| --- | --- |
| 36 | Error opening topo data file (no data) |
| 220 | Error reading terrain record |
| 400 | Height is missing (-9999) |
| 1000 | Tx record is not a Tx |
| 1001 | Rx record is not a Rx |
| 1002 | Tx frequency (1A) not present |
| 1003 | Tx frequency unit (1A1) not present |
| 1004 | Tx co-ordinates (4C) not present |
| 1005 | Tx height of site above sea level (4Z) not present |
| 1006 | Transmitter spectrum mask (7G) (frequency) is not present |
| 1007 | Transmitter spectrum mask (7G) (attenuation) is not present |
| 1008 | Tx power (8A) is not present |
| 1010 | Tx azimuth (9A) is not present |
| 1011 | Tx elevation (9B) is not present |
| 1012 | Tx polarization (9D) is not present |
| 1014 | Tx antenna height above ground (9Y) is not present |
| 1015 | Tx antenna gain (part of 9X) is not present |
| 1016 | Default Tx antenna radiation pattern (9X) is not present |
| 1020 | Rx frequency (1A) not present |
| 1021 | Rx frequency unit (1A1) not present |
| 1022 | Rx co-ordinates (4C) not present |
| 1023 | Rx height of site above sea level (4Z) not present |
| 1024 | Error in Tx channel spacing (7G3) |
| 1026 | Rx azimuth (9A) is not present |
| 1027 | Rx elevation (9B) is not present |
| 1028 | Rx polarization (9D) is not present |
| 1029 | Rx noise power level FKTB (9H) is not present |
| 1031 | Rx antenna height above ground (9Y) is not present |
| 1032 | Rx antenna gain (part of 9X) is not present |
| 1033 | Default Rx antenna radiation pattern (9X) is not present |
| 1037 | Error in Tx frequency (1A) |
| 1038 | Error in Tx frequency unit (1A1) |
| 1039 | Error in Tx frequency range (only 1 - 43.5 GHz are permissible) |
| 1040 | Error in Rx frequency (1A) |
| 1041 | Error in Rx frequency unit (1A1) |
| 1042 | Error in Rx frequency range (only 1 - 43.5 GHz are permissible) |
| 1043 | Error in Tx co-ordinates (4C) |
| 1044 | Error in Rx co-ordinates (4C) |
| 1045 | Error in Tx site height (4Z) |
| 1046 | Error in Rx site height (4Z) |
| 1047 | Error in Tx spectrum mask data (7G) |
| 1048 | Error in Rx selectivity mask data (7G) |
| 1049 | Error in Tx power (8A) |
| 1050 | Error in Tx ATPC (8B3) |
| 1051 | Error in Tx azimuth (9A) |
| 1052 | Error in Rx azimuth (9A) |
| 1053 | Error in Tx elevation (9B) |
| 1054 | Error in Rx elevation (9B) |
| 1055 | Error in (Rx) FkTB (9H) |
| 1056 | Error in Tx line losses (9L) |
| 1057 | Error in Rx line losses (9L) |
| 1058 | Error in Tx antenna height (9Y) |
| 1059 | Error in Rx antenna height (9Y) |
| 1060 | Error in Tx antenna gain (in 9X) |
| 1061 | Error in Rx antenna gain (in 9X) |
| 1062 | Tx position = Rx position; calculations not possible |
| 1070 | Receiver selectivity mask (7G) (frequency) is not present |
| 1071 | Receiver selectivity mask (7G) (attenuation) is not present |
| 1077 | PTx record is not a PTx |
| 1079 | PTx co-ordinates (4C) not present |
| 1080 | PTx height of site above sea level (4Z) not present |
| 1081 | PTx azimuth (9A) is not present |
| 1082 | PTx elevation (9B) is not present |
| 1083 | PTx polarization (9D) is not present |
| 1084 | PTx antenna height above ground (9Y) is not present |
| 1085 | PTx antenna gain (part of 9X) is not present |
| 1086 | Default PTx antenna radiation pattern (9X) is not present |
| 1090 | PRx record is not a PRx |
| 1091 | PRx co-ordinates (4C) not present |
| 1092 | PRx height of site above sea level (4Z) not present |
| 1093 | PRx azimuth (9A) is not present |
| 1094 | PRx elevation (9B) is not present |
| 1095 | PRx polarization (9D) is not present |
| 1096 | PRx antenna height above ground (9Y) is not present |
| 1097 | PRx antenna gain (part of 9X) is not present |
| 1098 | Default PRx antenna radiation pattern (9X) is not present |
| 1102 | Error in PTx co-ordinates (4C) |
| 1103 | Error in PRx co-ordinates (4C) |
| 1104 | Error in PTx site height (4Z) |
| 1105 | Error in PRx site height (4Z) |
| 1106 | Error in PTx azimuth (9A) |
| 1107 | Error in PRx azimuth (9A) |
| 1108 | Error in PTx elevation (9B) |
| 1109 | Error in PRx elevation (9B) |
| 1110 | Error in PTx line losses (9L) |
| 1111 | Error in PRx line losses (9L) |
| 1112 | Error in PTx antenna height (9Y) |
| 1113 | Error in PRx antenna height (9Y) |
| 1114 | Error in PTx antenna gain (in 9X) |
| 1115 | Error in PRx antenna gain (in 9X) |
| 1116 | Tx position = PRx position; calculations not possible |
| 1118 | PTx position = Rx position; calculations not possible |
| 1203 | Invalid Tx copolar radiation pattern (part of 9X) |
| 1204 | Invalid number of Tx copolar radiation pattern data (part of 9X) |
| 1205 | Invalid or missing Tx copolar antenna data (part of 9X) |
| 1206 | Invalid or missing Tx crosspolar radiation pattern (part of 9X) |
| 1207 | Invalid number of Tx crosspolar radiation pattern data (part of 9X) |
| 1208 | Invalid or missing Tx crosspolar antenna data (part of 9X) |
| 1209 | Invalid Rx copolar radiation pattern (part of 9X) |
| 1210 | Invalid number of Rx copolar radiation pattern data (part of 9X) |
| 1211 | Invalid or missing Rx copolar antenna data (part of 9X) |
| 1212 | Invalid or missing Rx crosspolar radiation pattern (part of 9X) |
| 1213 | Invalid number of Rx crosspolar radiation pattern data (part of 9X) |
| 1214 | Invalid or missing Rx crosspolar antenna data (part of 9X) |
| 1215 | Invalid PTx polarization (9D) |
| 1216 | Invalid PRx polarization (9D) |
| 1217 | Invalid Tx polarization (9D) |
| 1218 | Invalid Rx polarization (9D) |
| 1219 | Invalid PTx copolar radiation pattern (part of 9X) |
| 1220 | Invalid PRx copolar radiation pattern (part of 9X) |
| 1221 | Invalid number of PTx copolar radiation pattern data (part of 9X) |
| 1222 | Invalid number of PRx copolar radiation pattern data (part of 9X) |
| 1223 | Invalid or missing PTx copolar antenna data (part of 9X) |
| 1224 | Invalid or missing PRx copolar antenna data (part of 9X) |
| 1225 | Invalid or missing PTx crosspolar radiation pattern (part of 9X) |
| 1226 | Invalid or missing PRx crosspolar radiation pattern (part of 9X) |
| 1227 | Invalid number of PTx crosspolar radiation pattern data (part of 9X) |
| 1228 | Invalid number of PRx crosspolar radiation pattern data (part of 9X) |
| 1229 | Invalid or missing PTx crosspolar antenna data (part of 9X) |
| 1230 | Invalid or missing PRx crosspolar antenna data (part of 9X) |
| 1233 | Tx spectrum mask has less than 2 pairs of frequency and attenuation |
| 1234 | Rx selectivity mask has less than 2 pairs of frequency and attenuatio |
| 1235 | Last attenuation of Tx spectrum mask < 40 dB |
| 1236 | Last attenuation of Rx selectivity mask < 40 dB |
| 1237 | Error opening the result file |
| 1238 | Error writing data to the result file |
| 1241 | Invalid name of transmitter equipment class (7G1) |
| 1242 | Invalid name of receiver equipment class (7G1) |
| 1243 | No default transmitter mask data (7G) available |
| 1244 | No default receiver mask data (7G) available |
| 1245 | Could not find parameter file hcmrc |
| 1246 | Could not open registry key HKEY_LOCAL_MASHINE\Software\HCM |
| 1247 | Could not read topo path value from registry |
| 1248 | Error in Rx channel spacing (7G3) |