Interface documentation of HCM-FS4A (C++)

General information

The hcm4afs.dll is a dynamic link library written in C++.

Calling the DLLfunction:

Function HCM4AFS

Interference calculations. The main output parameter is threshold degradation (TD).

void HCM4AFS(double& I_O_AREA,

double& I_NFD,

double& I_MD,

double& I_I,

double& I_A_TOT,

double& I_TD,

int& I_Error,

const char* I_Record1,

const char* I_Record2,

const char* I_Record3,

const char* I_Record4,

const char* I_DLL_textfile,

const char* I_TopoPath,

const char* I_MorphoPath,

int I_Plz,

double I_TimePercentage,

double I_Press,

double I_Temp)

Output parameters:

- I_O_AREA 	overlapping area;

- I_NFD NFD 	value;

- I_MD MD 	value;

- I_I 		Interference level;

- I_A_TOT 	total attenuation;

- I_TD 		threshold degradation;

- I_Error 	error value.

Input parameters:

- I_Record1 Tx data according to ANNEX 2B;

- I_Record2 Rx or PRx data according to ANNEX 2B;

- I_Record3 PTx data according to ANNEX 2B or empty;

- I_Record4 Rx data according to ANNEX 2B or empty;

- I_DLL_textfile optional. Filename of an output file. Not calculated values appear in the file having values = -2222. This is not an error. This may happen when overlapping area = 0. In this case some parameters are not calculated and have value of NO DATA, which is -2222;

- I_TopoPath the path to the directory with the TOPO files;

- I_MorphoPath the path to the directory with the MORPHO files;

- int I_Plz normalized factor for surface admittance for horizontal (1) and vertical (2) polarizations, by default 1 (horizontal);

- double I_TimePercentage time percentages over the range 0.001  p  50%, by default 20.0%;

- double I_Press Average air pressure at the midpoint of the route, by default 1013.0 hPa

- double I_Temp Average annual air temperature at the midpoint of the route, by default 15.0°C

Requirements:

- Topological data Topological data in the HCM format. Mandatory. Data has to be placed into TOPO directory;

- Morphological data Morphological data in the HCM format. Optional. Data has to be placed into MORPHO directory.

Function HCM4AFS_ANNEX11

Calculations of co-ordination trigger, performed according to Annex 11 to the HCM-Agreement. This may help the administrations to decide if stations require co-ordination. The main output parameter is name of countries to coordinate with.

void HCM4AFS_ANNEX11(const double longitude,

const double latitude,

const double frequency_GHz,

const int h_asl,

const char* border_path,

char* outcome,

int& outlen,

int& error)

Output parameters:

- outcome list of countries to coordinate with or empty, if no coordination is required;

- outlen length of outcome;

- error Error code.

Input parameters:

- longitude Station longitude;

- latitude Station latitude;

- frequency_GHz Frequency of the station;

- h_asl Antenna height above sea level;

- border_path the path to the directory with the BORDER files.

Error Codes

| -1 | Unknown error |
| --- | --- |
| 0 | There are no errors |
| 1 | Invalid Type of entry |
| 2 | Invalid frequency |
| 3 | Invalid Frequency unit |
| 6 | Missing Geographical co-ordinates |
| 7 | Invalid Height of the station site above sea level |
| 8 | Missing frequency of transmitter spectrum mask or receiver selectivity mask |
| 9 | Invalid NFD1 |
| 10 | Invalid NFD2 |
| 11 | Invalid Channel spacing |
| 12 | Invalid Maximum permitted transmitter power |
| 13 | Invalid ATPC |
| 14 | Invalid Azimuth |
| 15 | Invalid Elevation |
| 16 | Invalid Polarization |
| 17 | Invalid Receiver noise level |
| 18 | Invalid Branches and line losses |
| 19 | Invalid Height of antenna above ground |
| 20 | Invalid antenna gain |
| 21 | Invalid Copolar radiation pattern |
| 22 | Invalid Number of copolar radiation pattern |
| 23 | Invalid Copolar antenna data or missing |
| 24 | Invalid Crosspolar radiation pattern |
| 25 | Invalid Number of crosspolar radiation pattern data |
| 26 | Invalid Crosspolar antenna data or missing |
| 27 | Missing attenuation of transmitter spectrum mask or receiver selectivity mask |
| 28 | Frequency is not present |
| 29 | Frequency unit is not present |
| 30 | Error in frequency range (only 1 - 39.5 GHz are permissible) |
| 31 | No height of the station site above sea level available |
| 32 | Error in transmitter spectrum mask or receiver selectivity mask |
| 33 | Transmitter power not present |
| 34 | Azimuth not present |
| 35 | Elevation not present |
| 36 | Error opening topo data file (no data) |
| 37 | Receiver noise power level not present |
| 38 | No antenna height above ground |
| 39 | Antenna gain not present |
| 40 | Antenna data (standard) not present |
| 42 | Equipment manufacturer name not present |
| 43 | Equipment type not present |
| 101 | Invalid geographical coordinates |
| 102 | Latitude is not in range of 0.0 - 90.0 |
| 103 | Longitude is not in range of 0.0 - 360.0 |
| 220 | Error reading terrain record |
| 361 | Polarization not available |
| 400 | Height is missing (-9999) |
| 401 | Error opening file (no data) |
| 402 | Error reading record |
| 406 | Distance = 0 |
| 501 | Tx and Rx masks do not overlap |
| 502 | Incorrect class name |
| 503 | Cannot find default mask |
| 601 | No borderline data found |
| 602 | Bad coordinates |
| 603 | Bad frequency |
| 604 | Error opening file |
| 605 | Error reading file |
| 999 | HCM record is empty |
| 1000 | Tx record is not a Tx |
| 1001 | Rx record is not a Rx |
| 1002 | Tx frequency (1A) not present |
| 1003 | Tx frequency unit (1A1) not present |
| 1004 | Tx co-ordinates (4C) not present |
| 1005 | Tx height of site above sea level (4Z) not present |
| 1006 | Transmitter spectrum mask (7G) (frequency) is not present |
| 1007 | Transmitter spectrum mask (7G) (attenuation) is not present |
| 1008 | Tx power (8A) is not present |
| 1010 | Tx azimuth (9A) is not present |
| 1011 | Tx elevation (9B) is not present |
| 1012 | Tx polarization (9D) is not present |
| 1014 | Tx antenna height above ground (9Y) is not present |
| 1015 | Tx antenna gain (part of 9X) is not present |
| 1016 | Default Tx antenna radiation pattern (9X) is not present |
| 1020 | Rx frequency (1A) not present |
| 1021 | Rx frequency unit (1A1) not present |
| 1022 | Rx co-ordinates (4C) not present |
| 1023 | Rx height of site above sea level (4Z) not present |
| 1024 | Error in Tx channel spacing (7G3) |
| 1026 | Rx azimuth (9A) is not present |
| 1027 | Rx elevation (9B) is not present |
| 1028 | Rx polarization (9D) is not present |
| 1029 | Rx noise power level FKTB (9H) is not present |
| 1031 | Rx antenna height above ground (9Y) is not present |
| 1032 | Rx antenna gain (part of 9X) is not present |
| 1033 | Default Rx antenna radiation pattern (9X) is not present |
| 1037 | Error in Tx frequency (1A) |
| 1038 | Error in Tx frequency unit (1A1) |
| 1039 | Error in Tx frequency range (only 1 - 43.5 GHz are permissible) |
| 1040 | Error in Rx frequency (1A) |
| 1041 | Error in Rx frequency unit (1A1) |
| 1042 | Error in Rx frequency range (only 1 - 43.5 GHz are permissible) |
| 1043 | Error in Tx co-ordinates (4C) |
| 1044 | Error in Rx co-ordinates (4C) |
| 1045 | Error in Tx site height (4Z) |
| 1046 | Error in Rx site height (4Z) |
| 1047 | Error in Tx spectrum mask data (7G) |
| 1048 | Error in Rx selectivity mask data (7G) |
| 1049 | Error in Tx power (8A) |
| 1050 | Error in Tx ATPC (8B3) |
| 1051 | Error in Tx azimuth (9A) |
| 1052 | Error in Rx azimuth (9A) |
| 1053 | Error in Tx elevation (9B) |
| 1054 | Error in Rx elevation (9B) |
| 1055 | Error in (Rx) FkTB (9H) |
| 1056 | Error in Tx line losses (9L) |
| 1057 | Error in Rx line losses (9L) |
| 1058 | Error in Tx antenna height (9Y) |
| 1059 | Error in Rx antenna height (9Y) |
| 1060 | Error in Tx antenna gain (in 9X) |
| 1061 | Error in Rx antenna gain (in 9X) |
| 1062 | Tx position = Rx position; calculations not possible |
| 1067 | Antenna co-polar data for the respective polarisation is not available |
| 1068 | Antenna cross-polar data for the respective polarisation is not available |
| 1070 | Receiver selectivity mask (7G)(frequency) is not present |
| 1071 | Receiver selectivity mask (7G)(attenuation) is not present |
| 1075 | Rx equipment manufacturer name (7H) not present |
| 1077 | PTx record is not a PTx |
| 1079 | PTx co-ordinates (4C) not present |
| 1080 | PTx height of site above sea level (4Z) not present |
| 1081 | PTx azimuth (9A) is not present |
| 1082 | PTx elevation (9B) is not present |
| 1083 | PTx polarization (9D) is not present |
| 1084 | PTx antenna height above ground (9Y) is not present |
| 1085 | PTx antenna gain (part of 9X) is not present |
| 1086 | Default PTx antenna radiation pattern (9X) is not present |
| 1090 | PRx record is not a PRx |
| 1091 | PRx co-ordinates (4C) not present |
| 1092 | PRx height of site above sea level (4Z) not present |
| 1093 | PRx azimuth (9A) is not present |
| 1094 | PRx elevation (9B) is not present |
| 1095 | PRx polarization (9D) is not present |
| 1096 | PRx antenna height above ground (9Y) is not present |
| 1097 | PRx antenna gain (part of 9X) is not present |
| 1098 | Default PRx antenna radiation pattern (9X) is not present |
| 1102 | Error in PTx co-ordinates (4C) |
| 1103 | Error in PRx co-ordinates (4C) |
| 1104 | Error in PTx site height (4Z) |
| 1105 | Error in PRx site height (4Z) |
| 1106 | Error in PTx azimuth (9A) |
| 1107 | Error in PRx azimuth (9A) |
| 1108 | Error in PTx elevation (9B) |
| 1109 | Error in PRx elevation (9B) |
| 1110 | Error in PTx line losses (9L) |
| 1111 | Error in PRx line losses (9L) |
| 1112 | Error in PTx antenna height (9Y) |
| 1113 | Error in PRx antenna height (9Y) |
| 1114 | Error in PTx antenna gain (in 9X) |
| 1115 | Error in PRx antenna gain (in 9X) |
| 1116 | Tx position = PRx position)); calculations not possible |
| 1118 | PTx position = Rx position)); calculations not possible |
| 1203 | Invalid Tx copolar radiation pattern (part of 9X) |
| 1204 | Invalid number of Tx copolar radiation pattern data (part of 9X) |
| 1205 | Invalid or missing Tx copolar antenna data (part of 9X) |
| 1206 | Invalid or missing Tx crosspolar radiation pattern (part of 9X) |
| 1207 | Invalid number of Tx crosspolar radiation pattern data (part of 9X) |
| 1208 | Invalid or missing Tx crosspolar antenna data (part of 9X) |
| 1209 | Invalid Rx copolar radiation pattern (part of 9X) |
| 1210 | Invalid number of Rx copolar radiation pattern data (part of 9X) |
| 1211 | Invalid or missing Rx copolar antenna data (part of 9X) |
| 1212 | Invalid or missing Rx crosspolar radiation pattern (part of 9X) |
| 1213 | Invalid number of Rx crosspolar radiation pattern data (part of 9X) |
| 1214 | Invalid or missing Rx crosspolar antenna data (part of 9X) |
| 1215 | Invalid PTx polarization (9D) |
| 1216 | Invalid PRx polarization (9D) |
| 1217 | Invalid Tx polarization (9D) |
| 1218 | Invalid Rx polarization (9D) |
| 1219 | Invalid PTx copolar radiation pattern (part of 9X) |
| 1220 | Invalid PRx copolar radiation pattern (part of 9X) |
| 1221 | Invalid number of PTx copolar radiation pattern data (part of 9X) |
| 1222 | Invalid number of PRx copolar radiation pattern data (part of 9X) |
| 1223 | Invalid or missing PTx copolar antenna data (part of 9X) |
| 1224 | Invalid or missing PRx copolar antenna data (part of 9X) |
| 1225 | Invalid or missing PTx crosspolar radiation pattern (part of 9X) |
| 1226 | Invalid or missing PRx crosspolar radiation pattern (part of 9X) |
| 1227 | Invalid number of PTx crosspolar radiation pattern data (part of 9X) |
| 1228 | Invalid number of PRx crosspolar radiation pattern data (part of 9X) |
| 1229 | Invalid or missing PTx crosspolar antenna data (part of 9X) |
| 1230 | Invalid or missing PRx crosspolar antenna data (part of 9X) |
| 1233 | Tx spectrum mask has less than 2 pairs of frequency and attenuation |
| 1234 | Rx selectivity mask has less than 2 pairs of frequency and attenuation |
| 1235 | Last attenuation of Tx spectrum mask < 40 dB |
| 1236 | Last attenuation of Rx selectivity mask < 40 dB |
| 1237 | Error opening the result file |
| 1238 | Error writing data to the result file |
| 1241 | Invalid name of transmitter equipment class (7G1) |
| 1242 | Invalid name of receiver equipment class (7G1) |
| 1243 | No default transmitter mask data (7G) available |
| 1244 | No default receiver mask data (7G) available |
| 1245 | Could not find parameter file hcmrc |
| 1246 | Could not open registry key HKEY_LOCAL_MASHINE\\Software\\HCM |
| 1247 | Could not read topo path value from registry |
| 1248 | Error in Rx channel spacing (7G3) |