Interface documentation of HCM-MS (C++) ver. 2.0

General information & Cross-Platform Support	1

Exported Functions	2

Input Structure: PointToPoint_Input_Parameters	3

Output Structure: PointToPoint_Output_Parameters	6

Info Flags	9

Error Codes	10

# General information& Cross-Platform Support

Cross-platform: This library can be compiled for Windows (.dll) and Linux(.so). Uses conditional export macros (__declspec(dllexport) on Win32, default visibility on POSIX).

Compatibility: C-compatible extern "C" interface. Usable from C++, C#, Python (ctypes/cffi), Rust, Go, etc.

Loading the DLL: Supports both static (implicit) and dynamic (explicit) loading.

Static: Link against the generated import library. Functions resolve at process startup.

Dynamic: Load at runtime via LoadLibrary/GetProcAddress (Windows) or dlopen/dlsym (Linux). Allows hot-swapping the library without recompiling the host application.

# Exported Functions

| Function Name | Full Signature | Return Type | Description |
| --- | --- | --- | --- |
| PointToPoint | void PointToPoint(PointToPoint_Input_Parameters &input, PointToPoint_Output_Parameters &output) | void | Performs point-to-point field strength calculation. |
| LineCalc | void LineCalc(PointToPoint_Input_Parameters &input, PointToPoint_Output_Parameters &output) | void | Performs point-to-line (border) calculation. |
| CbrCalc | void CbrCalc(PointToPoint_Input_Parameters &input, PointToPoint_Output_Parameters &output) | void | Performs cross-border range calculation. |
| GetInfoString | void GetInfoString(char* outputString) | void | Returns DLL version string. Output buffer must be at least 20 characters. |
| GetStructureSizes | void GetStructureSizes(int* sizeIn, int* sizeOut) | void | Returns sizeof(PointToPoint_Input_Parameters) and sizeof(PointToPoint_Output_Parameters) for compatibility checks. |
| GetErrorMessage | void GetErrorMessage(int errorCode, char* outputString, int bufferSize) | void | Returns human-readable error description for given error code. Output is null-terminated and truncated to bufferSize - 1. |
| GetInfoMessage | void GetInfoMessage(int infoCode, char* outputString, int bufferSize) | void | Returns human-readable description for given info flag codes (1–18). Output is null-terminated and truncated to bufferSize - 1. |

# Input Structure: PointToPoint_Input_Parameters

const int HCM4A_NO_INPUT_INT = -999;

const double HCM4A_NO_INPUT_DOUBLE = -999.0;

| Field Name | Data Type | Default / Valid Range | Description |
| --- | --- | --- | --- |
| Tx_Longitude | double | -180.0 … +180.0 | Transmitter longitude in decimal degrees. Positive = East, negative = West. |
| Tx_Latitude | double | -89.9 … +89.9 | Transmitter latitude in decimal degrees. Positive = North, negative = South. |
| Rx_Longitude | double | -180.0 … +180.0 | Receiver longitude in decimal degrees. |
| Rx_Latitude | double | -89.9 … +89.9 | Receiver latitude in decimal degrees. |
| Tx_Power | double | -100.0 … +100.0 dBW | Transmitter effective radiated power in dBW. |
| Tx_H_Ant | double | 0.0 … 150.0 m | Transmitter antenna height above ground level. |
| Rx_H_Ant | double | 0.0 … 150.0 m | Receiver antenna height above ground level. |
| Tx_Serv_area | double | 0.0 … 150.0 km | Transmitter service area radius. >0 indicates mobile transmitter. |
| Rx_Serv_area | double | 0.0 … 150.0 km | Receiver service area radius. >0 indicates mobile receiver. |
| Tx_Freq | double | >0 MHz | Transmitter frequency in MHz. Used for propagation model selection. |
| Rx_Freq | double | >0 MHz | Receiver frequency in MHz. Usually equal to Tx_Freq. |
| Tx_Azi | double | 0.0 … 360.0° | Transmitter antenna azimuth (horizontal direction). |
| Rx_Azi | double | 0.0 … 360.0° | Receiver antenna azimuth. |
| Tx_Elev | double | -90.0 … +90.0 ° | Transmitter antenna elevation angle. |
| Rx_Elev | double | -90.0 … +90.0 ° | Receiver antenna elevation angle. |
| Rx_Ant_gain | double | -50.0 … +50.0 | Receiver antenna gain. |
| Depol_loss | double | any | Depolarization loss in dB |
| PointDistance | double | 0.03 … 1.0 km | Step size for terrain profile sampling. Auto-corrected to 0.1 if out of range. |
| Perm_FS_input | double | HCM4A_NO_INPUT_DOUBLE or user value | User-provided permissible field strength. |
| Max_CBR_D_input | double | HCM4A_NO_INPUT_INT_DOUBLE or user value | User-provided maximum cross-border range. Reserved for future use. |
| Cor_fact_frequ_diff | double | any | Correction factor for frequency difference (reserved). |
| Tx_Type_of_Ant | int | 0 or 1 | Transmitter antenna type. |
| Rx_Type_of_Ant | int | 0 or 1 | Receiver antenna type. |
| Chan_occup | int | 0 or 1 | Channel occupation flag |
| C_Mode | int | -11 … 12 | Calculation mode selector. |
| D_to_border | int | 0 … 999 km | Distance to borderline for LineCalc/CbrCalc. |
| MaxCalcDistanceKm | int | >0 | Maximum allowed Tx–Rx distance. Default 1000. |
| useAnnex1 | int | 0 or 1 | If 1 and frequency in Annex 1 range, use table-defined Perm_FS and CBR_D. |
| Rx_H_Site_Ant | int | HCM4A_NO_INPUT_INT or meters above sea level | Receiver site height above sea level. If -999 or Rx_Serv_area>0, fetched from terrain DB. |
| Tx_H_Site_Ant | int | HCM4A_NO_INPUT_INT or meters above sea level | Transmitter site height above sea level. Same logic as Rx_H_Site_Ant. |
| Tx_Ant_typ_H | char[8] | "000ND00" default | Transmitter horizontal antenna pattern code. |
| Tx_Ant_typ_V | char[8] | "000ND00" default | Transmitter vertical antenna pattern code. |
| Rx_Ant_typ_H | char[8] | "000ND00" default | Receiver horizontal antenna pattern code. |
| Rx_Ant_typ_V | char[8] | "000ND00" default | Receiver vertical antenna pattern code. |
| Tx_Desig_of_emis | char[10] | e.g. "80M0G7WET" | Transmitter emission designation. |
| Rx_Desig_of_emis | char[10] | e.g. "80M0G7WET" | Receiver emission designation. |
| Sea_temperature | char[1] | 'W', 'I', or 'C' | Sea surface temperature category: Warm, Intermediate, Cold. Default 'W'. |
| Border_path | char[256] | absolute path | Directory containing border files (*.all, *.cbr). |
| Topo_path | char[256] | absolute path | Directory containing terrain height data files. |
| Morpho_path | char[256] | absolute path | Directory containing morphological data files. |
| Debug_file | char[256] | optional path | Path for debug log output. If empty, no debug file is written. |
| Land_from | char[4] | 3-letter country code | Country code of transmitter side (e.g. "ERI"). |
| Land_to | char[4] | 3-letter country code | Country code of receiver side (e.g. "ETH"). |

# Output Structure: PointToPoint_Output_Parameters

| Field Name | Data Type | Unit / Range | Description |
| --- | --- | --- | --- |
| Distance | double | km | distance between calculated Tx and Rx coordinates. |
| Dir_Tx_Rx | double | 0…360 | Azimuth from Tx to Rx |
| Dir_Rx_Tx | double | 0…360 | Azimuth from Rx to Tx. |
| Tx_TCA | double | -90…+90 | Transmitter terrain clearance angle. |
| Rx_TCA | double | -90…+90 | Receiver terrain clearance angle. |
| Tx_TCA_corr | double | dB | Correction factor for Tx clearance angle |
| Rx_TCA_corr | double | dB | Correction factor for Rx clearance angle. |
| V_angle_Tx_Rx | double | degrees | Vertical angle from Tx to Rx (elevation). |
| V_angle_Rx_Tx | double | degrees | Vertical angle from Rx to Tx. |
| H_diff_angle_Tx_Rx | double | degrees | Horizontal deviation between antenna boresight and Rx direction. |
| V_diff_angle_Tx_Rx | double | degrees | Vertical deviation between antenna boresight and Rx direction. |
| Tx_ant_corr | double | dB | Antenna pattern correction for Tx |
| Tx_ant_type_corr | double | dB | Antenna type correction |
| Power_to_Rx | double | dBW | Effective power in direction of Rx |
| Free_space_FS | double | dBµV/m | Free-space field strength |
| Calculated_FS | double | dBµV/m | Final computed field strength at Rx location. |
| Perm_FS | double | dBµV/m | Permissible field strength |
| Perm_FS_from_table | double | dBµV/m | Table-defined permissible FS. |
| Table_CBR_D | double | km | Table-defined maximum cross-border range for given frequency band. |
| ERP_ref_Tx | double | dBW | Reference transmitter ERP for coordination purposes. |
| Time_percentage | double | % | Time percentage for which field strength is not exceeded (1, 10, or 50). |
| Prot_margin | double | dB | Protection margin |
| Tx_Longitude_Calc | double | decimal degrees | Final Tx longitude. |
| Tx_Latitude_Calc | double | decimal degrees | Final Tx latitude. |
| Rx_Longitude_Calc | double | decimal degrees | Final Rx longitude |
| Rx_Latitude_Calc | double | decimal degrees | Final Rx latitude |
| Heff_Tx | double | m | Effective height of Tx antenna above average terrain. |
| Heff_Rx | double | m | Effective height of Rx antenna above average terrain. |
| Heff | double | m | effective height |
| D_sea_calculated | double | km | Length of path segment over sea, derived from morphological data. |
| Dh | double | m | Terrain irregularity parameter |
| Dh_corr | double | dB | Correction factor for terrain irregularity. |
| Land_FS | double | dBµV/m | Field strength over land path component. |
| Sea_FS | double | dBµV/m | Field strength over sea path component. |
| Corr_delta_f | double | dB | Correction for frequency difference. |
| Delta_frequency | double | kHz | Frequency difference Tx–Rx. |
| Rx_ant_corr | double | dB | Antenna pattern correction for Rx. |
| H_diff_angle_Rx_Tx | double | degrees | Horizontal deviation for Rx antenna. |
| V_diff_angle_Rx_Tx | double | degrees | Vertical deviation for Rx antenna. |
| Depol_loss_calc | double | dB | Calculated depolarization loss. |
| Rx_ant_type_corr | double | dB | Rx antenna type correction. |
| ProfilePoints | int | count | Number of points in terrain profile between Tx and Rx. |
| AnnexInFreqRange | int | 0 or 1 | 1 in case, if useAnnex1 is 1 and all conditions for useAnnex1 are met (permissible level is taken from Annex 1) |
| Info[20] | int[20] | 0 or 1 | Boolean flags for calculation notes (see below). |
| Tx_H_Site_Ant | int | m | Final Tx site height above sea level (from input or terrain DB). |
| Rx_H_Site_Ant | int | m | Final Rx site height above sea level. |
| HCM_error | int | 0 = success, >0 = error code | Error status. 0 means calculation completed successfully. |
| Version | char[256] | null-terminated string | version: "Version 1.24 One2One", "LineCalc", or "CbrCalc". |

# Info Flags

| Index | Flag Name (Conceptual) | Meaning |
| --- | --- | --- |
| 1 | TxHeightFromDB | Tx site height was not provided or Tx is mobile; height taken from terrain database. |
| 2 | TxHeightDiffMinor | Tx site height from DB differs from input by ≤10%; values may have minor inaccuracy. |
| 3 | TxHeightDiffMajor | Tx site height from DB differs from input by >10%; calculated values may be extremely wrong. |
| 4 | PermFSNotFromAgreement | Permissible field strength not determined by bilateral agreement; user must provide Perm_FS_input. |
| 5 | PermFSUserOverride | User-provided permissible field strength value is used instead of table value. |
| 6 | CBR_D_UserOverride | User-provided maximum cross-border range is used (currently not active). |
| 7 | OverlappingServiceAreas | Tx and Rx service areas overlap; field strength set to 999.9 dBµV/m (invalid result marker). |
| 8 | RxHeightFromDB | Rx site height taken from terrain database. |
| 9 | RxHeightDiffMinor | Rx site height from DB differs from input by ≤10%. |
| 10 | RxHeightDiffMajor | Rx site height from DB differs from input by >10%; results may be unreliable. |
| 11 | FreeSpaceShortDistance | Free-space propagation model used because distance is less than 1 km. |
| 12 | FreeSpaceFresnelClear | Free-space model used because 1st Fresnel zone is unobstructed. |
| 13 | SeaDistanceClamped | Calculated sea path length exceeded total distance; clamped to total distance. |
| 14 | FreqDiffCorrectionUsed | User-provided frequency-difference correction factor is applied. |
| 15 | FreqDiffOutOfRange | Default correction value 82 dB used due to out-of-range frequency difference. |
| 16 | MorphoDataMissingSea | Sea path length set to 0 due to missing morphological data. |
| 17 | TxChanSpacingDefault | Default channel spacing 25 kHz used for Tx. |
| 18 | Band380_400Correction | Additional correction factors for 380–400 MHz band applied per regional agreement. |

# Error Codes

| Code | Error Message |
| --- | --- |
| 0 | No error |
| 36 | Error opening terrain- or morphological data file (data not available) |
| 38 | Error opening border data file (data not available) |
| 41 | Error get height for Tx (Calculated coord) (data not available) |
| 42 | Error get height for Rx (Calculated coord) (data not available) |
| 200 | Error in longitude (in 'Point_height' or 'Point_type' subroutine) |
| 210 | Error in latitude (in 'Point_height' or 'Point_type' subroutine) |
| 220 | Error reading record (in 'Point_height' or 'Point_type' subroutine) |
| 300 | Latitude is not in range of 0.0 - 90.0 (in 'Point_height' or 'Point_type' subroutine) |
| 400 | Height is missing (-9999) (in 'Point_height' subroutine) |
| 1000 | Distance between Tx and Rx = 0. Calculations not possible |
| 1001 | Error in geographical coordinates (Tx longitude, degrees) |
| 1002 | Error in geographical coordinates (Tx longitude, minutes) |
| 1003 | Error in geographical coordinates (Tx longitude, seconds) |
| 1004 | Error in geographical coordinates (Tx longitude, E/W) |
| 1005 | Error in geographical coordinates (Tx latitude, degrees) |
| 1006 | Error in geographical coordinates (Tx latitude, minutes) |
| 1007 | Error in geographical coordinates (Tx latitude, seconds) |
| 1008 | Error in geographical coordinates (Tx latitude, N/S) |
| 1009 | Error in Tx antenna height |
| 1010 | Error in transmitting frequency value |
| 1011 | Error in transmitting frequency unit |
| 1012 | Error in radius of service area of Tx |
| 1013 | Error in input value height of Tx site above sea level |
| 1014 | Error in geographical coordinates (Rx longitude, degrees) |
| 1015 | Error in geographical coordinates (Rx longitude, minutes) |
| 1016 | Error in geographical coordinates (Rx longitude, seconds) |
| 1017 | Error in geographical coordinates (Rx longitude, E/W) |
| 1018 | Error in geographical coordinates (Rx latitude, degrees) |
| 1019 | Error in geographical coordinates (Rx latitude, minutes) |
| 1020 | Error in geographical coordinates (Rx latitude, seconds) |
| 1021 | Error in geographical coordinates (Rx latitude, N/S) |
| 1022 | Error in Rx antenna height |
| 1023 | Error in reception frequency value |
| 1024 | Error in reception frequency unit |
| 1025 | C_mode is out of range |
| 1026 | Error in input value of permissible field strength |
| 1027 | Error in input value of maximum cross border range |
| 1028 | The distance is more than acceptable. Calculations are impossible |
| 1029 | Error in radius of Rx service area |
| 1030 | Error in input value Rx site height above sea level |
| 1031 | Error in Tx elevation |
| 1032 | Error in Tx azimuth |
| 1033 | Error in type of Tx antenna (E/I) |
| 1034 | Error in power |
| 1035 | Error in input value of distance over sea |
| 1036 | The 'xxx.ALL' borderline file for Tx is missing |
| 1037 | The 'xxx.ALL' borderline file for Rx is missing |
| 1038 | Error in type of antenna (TR20-08) |
| 1039 | Error in input data of correction factor according frequency difference |
| 1040 | Channel spacing outside definition range (Rx) |
| 1041 | Channel spacing outside definition range (Tx) |
| 1042 | Error in Rx elevation |
| 1043 | Error in Rx azimuth |
| 1044 | Error in Rx type of antenna E/I |
| 1045 | Error in gain of Rx antenna |
| 1046 | Error in input data of depolarization loss |
| 1047 | Distance to borderline is too long |
| 1048 | Selected line data not available |
| 1049 | Error in line data |
| 1050 | non strict HCM Cmode and important technical data missing |
| 2000 | wrong Figure_frequency (from Get_figure_FS_value) |
| 2001 | wrong Time_percentage (from Get_figure_FS_value) |
| 2002 | wrong Sea_temperature (from Get_figure_FS_value) |
| 2003 | wrong Figure_Heff (from Get_figure_FS_value) |
| 2004 | wrong Figure_distance (from Get_figure_FS_value) |
| 2005 | wrong Get_figure_FS_value (error in curves) |